## Kolb Mark 3 Xtra check list

BEFORE ENTERING THE AIRPLANE

- Make an exterior inspection of the airplane

BEFORE STARTING ENGINE

- Check fuel

- Check seat belts—adjust & lock

- Brakes—test for firmness

STARTING ENGINE

- Brakes – set

- Master “On”

- Fuel pump “On” for 10 seconds,then Off

- Throttle  “Closed”

- Choke “ON”

- Prop “Clear”

- Engage “Starter”

- Check Oil Pressure is rising

- 1200 RPM’s until engine runs smoothl

BEFORE TAKE OFF

(1)Warm up engine until oil temp reaches 59*

(2) Mag check at 2000 rpm’s—mag drop no more

Than 100 rpm

(3)CHT 212* before full take off power

- Flight controls – Free and correct movement

- Elevator trim – Full Forward

- Cabin Doors – Closed and latched

CHT   --   356* CRUISE          392* CLIMB

OIL TEMP CRUISE---  176- 212*   (244 MAX)

TAKE OFF

- Fuel pump – “ON”

- Slowly advance throttle to full

- Climb at 60 mph – 3300 rpm

WARNING – IF LOSS OF POWER

KEEP NOSE DOWN !

CRUISE

- Fuel pump “OFF”

- Cruise at 2800 rpm

LANDING

(1)Fuel pump “ON”

(2) Flaps—as needed

(3) Air speed—60 mph

NORMAL LANDING

- 60 mph down to the ground

- Do not flair until just above the ground

WARNING ! ANY SUDDEN LOSS OF POWER

KEEP NOSE DOWN !

AFTER LANDING

- Flaps up

- Fuel pump – “OFF”

SECURING AIRCRAFT

- Radios –“OFF”

- Strobes and lights  -- “OFF”

- MAGS – “OFF”

- MASTER –“OFF”

(5) Fill out LOG BOOK