Operator’s Manual – Kolb Mark III

Builder: Steven Green - 2001

N58SG

Contents

Setup and preparation

Before Starting Checklist

Before Takeoff Checklist

Takeoff

Cruise Checklist

Before Landing Checklist

Landing

Shutdown Checklist

Limitations

Emergency Operations

Setup:

See instructions

Preflight Inspection Checklist:

See laminated checklist

Starting Checklist:

Fuel level: Check

Starting Circuit Fuse: Insert, if removed for safety between flights

Document data

EIS Hours

Hobbs Hours

Fuel level

Accessories: Off

Brakes: Held

Master Switch: On

Electric Fuel Pump: on for 10 seconds

Mags: Both on

Area: Clear

Choke: On

Throttle: Closed

Starter: Engage (10 second maximum, x 2, followed by 2 minute cool-down)

When started: Slowly release choke/ advance throttle to 1500rpm

Oil Pressure: Check

Accessories: On

Intercom: On

Headset volume: Full

Radio and GPS: On

Strobes: On

ATIS: Record

Altimeter: Set

BRS Pin: Remove

Before Takeoff Checklist:

ATIS: Confirm

Runway number: Confirm

Controls: Free and correct

Trim: Set for takeoff

Headset volume-full

Brake: Held

Throttle: 3200rpm

Mags: Check individually – Maximum drop=100rpm / Difference=50rpm

Throttle: 1600rpm

Oil Temp: Minimum 120 Degrees Fahrenheit

Fuel level: Check

BRS Pin: Check removed

Takeoff:

Clearance for takeoff: Read back

Align with runway

Apply back pressure

Advance throttle smoothly to “Full”

Back pressure: Slowly relax to raise tail

Rotate at 40 kts.

Establish and maintain safe climb airspeed

Vx – 45kts.

Vy – 55kts.

Cruise Checklist:

Attitude: Level

Throttle:

5500rpm max

4800rpm economy

4100rpm pattern

Trim: Adjust for neutral elevator pressure

Before Landing Checklist:

Check ATIS

Clearance for landing: Read back

Throttle: 4100rpm

Trim: None

Pattern airspeed: 50kts. - Minimum

Approach airspeed: 50kts.  – Minimum

Flaps: As needed

Landing:

Approach Airspeed: 50kts. – Minimum

Flare: 1 to 2 feet

Rollout: Maintain runway alignment

Brakes: As needed

Shutdown Checklist:

Strobes: Off

Radio and GPS: Off

Intercom: Off

Accessories: Off

Mags: Off

Master: Off

BRS Pin: Insert

Starting Circuit Fuse: Remove for safety

Emergency Checklists:

Power loss on takeoff:

Trim and Maintain safe airspeed - 50 kts.

Below 1000 feet- Maintain heading w/in 45 degrees

Above 1000 feet- Gentle turns toward suitable field

Mags- Check Both On

Flaps: Apply as needed once field is made

Communicate: Mayday, location, nature of emergency if time permits

Landing Airspeed: Minimum 50kts.

Power loss in cruise:

Trim and maintain safe airspeed 40-50 kts.

Suitable Field Available:

Select and steer for suitable field

Mags: Check Both On

See ‘Power loss on takeoff’

Suitable field unavailable:

Trim and maintain safe airspeed

At Safe Altitude, (500ft. minimum,) slow to 40kts.

Mags: Off

BRS: Deploy

Limitations:

Stall speeds (IAS) -

Vs0 – 25kts.

Vs1 – 30kts.

Best Glide – 40-50kts.

Vx – 45kts.

Vy – 50kts.

Mfe – 55kts.

Va – 70kts.

Vne – 80kts.

| Horsepower: | 100 hp |  |  |
| --- | --- | --- | --- |
| Fuel Capacity: | 20 gal |  |  |
| Cruise: | 70 kts |  |  |
| Stall: | 30 kts (Vs1) |  |  |
| Climb: | 1,100 ft/min |  |  |
| Takeoff Distance: | 175 ft |  |  |
| Landing Distance: | 250 ft |  |  |
| - Kolb Mark III Specifications |  |  |  |
| Gross Weight: | 1,050 lbs |  |  |
| Empty Weight: | 591 lbs |  |  |
| Useful Load: | 459 lbs |  |  |
| Wingspan: | 30 ft |  |  |
| Length: | 24 ft |  |  |
| Height: | 6.33 ft |  |  |