WEIGHT AND BALANCE REPORT

Aero Vodochody L-29		N8164C		S/N093734

B

256.26

114.45

80.31

7.28

38.98

169.29

11

| Name |  |
| --- | --- |
| Empty Plane | E |
| Fuel Main Tanks | M |
| Pilot Front Cockpit | P1 |
| Pilot Rear Cockpit | P2 |
| Aux Fuel Tanks | A |
| 15% Minimum Fuel – 150l | Mm |
| Ballast | B |

20.1

25%

163.58

P2

133.62

M

E

A

Mm

80

MAC

P1

| Name | Weight | Arm | Moment | CG Aft of Datum | %MAC |
| --- | --- | --- | --- | --- | --- |
| Main Gear | 4850 | 31.72 | 153842 |  |  |
| Nose Gear | 240 | -90.58 | -21739 |  |  |
| Ballast | 65 | -163.58 | -10633 |  |  |
| Empty Weight | 5155 |  | 121470 | 23.56 |  |
| Pilot Front Cockpit | 198 | -83.58 | -16549 |  |  |
| Pilot Rear Cockpit | 198/0 | -29.96 | -5932/0 |  |  |
| Fuel Main Tanks 39.6/264gal*6.7 | 261/1769 | 22.72 | 5930/40192 |  |  |
| Aux Fuel Tanks 79gal*6.7 | 0/529 | 20.28 | 0/10728 |  |  |
|  | 5812/7651 |  | 104919/155841 | 18.05/20.37 | 22.48/25.36 |

Aircraft weight with parachutes installed and gear extended – Retracting the gear moves CG forward 0.7 - 1.1%

All weights are in pounds and all dimensions are in inches.  Fuel weight 6.7lb/gal.

Approved CG Range 20.5% - 25.5% MAC (16.46” – 20.47” Aft of Datum)

Most Forward CG - 2 Pilots, Min Fuel / Most Aft CG – 1 Pilot, Full Fuel

______________________________

0

191.02

187.05

186.29

179.92