VORTEX GENERATORS

Allegro 2000 first test flight with Stolspeed VGs.

3 flights, total time about 2.5 hrs flown with instructor Michael Peare

Density alt 2,600'

CG close to forward limit

Weight 490 KG

Pictures and info for VGs at www.stolspeed.com

Test 1 - VGs half span

Test 2 - VGs full span

Test 3  - VGs full span but leave gap of 80cms at wing root

|  | POWER OFF |  |  | POWER ON 3000 RPM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (Flap setting) | 0 | 15 | 48 | 0 | 15 | 48 |
| NO VG | 45 | 40 | 36 | 44 | 38 | 35 |
| HALF SPAN VG | 43 | 36 | 36 | 43 | 37 | 34 |
| FULL SPAN VG | 41 | 36 | 34 | 38 | 36 | 34 |
| Stall speed reduced | 4 | 4 | 2 | 6 | 2 | 1 |
| Airspeeds in knots – Dynon and ASI both calibrated |  |  |  |  |  |  |

No VGs

Aircraft breaks and drops right wing. Significant wing drop with power on.

Little buffet just before stall. Ailerons have adverse yaw close to stall. Needs some rudder to correct wing drop.

Ailerons a little heavy.

Test 1 - VGs half span and VGs on vertical stabiliser

Ailerons lighter, more effective.

A lot of buffet warning

Power off - no break, no wing drop. Stick full back, aircraft nose high and buffets

Power on - mild wing drop, quick recovery.

Can be rolled 30 left 30 right a few knots below stall. Stable.

Ailerons can be used to roll level after stall.

Can fly 40 degree banked turn few knots below stall. Aircraft buffets and purpoises but does not break.

Small reduction stall speed except power off reduced 4 kts @ half flap

Test 2 - VGs full span

Ailerons same as test 1.

Reduced buffet

Power off - mild wing drop, quick recovery. At zero flap, no break, descends 500'/min

Power on - break and mild wing drop, quick recovery

Significant reduction in clean stall speed, power on/off.

Aircraft can be trimmed to fly at 38 knots full flap with very high nose angle. Stable.

Test 3 - removed VGs at the wing root (approx span of the elevator) to try to get the strong buffet in test 1 with half span VGs. Made no difference.

Notes.

Factory stall warning gave good warning under all tests. For example power off, full flap the stall warn sounds @ 42, buffet @ 37 and stall 34

Right wing drop even from left turn with left rudder 34 kts 3,800 RPM

Increasing RPM to 4,000 full flap, stall speed same, 34 kts, wing drop not significantly worse with more power (very different to standard config with no VGs)

Stall speeds in POH say 43 - 39 - 35 kts

Conclusion

The VGs on the tail did not seem to give a noticeable difference in handling.

Results broadly similar to many aircraft i.e. large difference in clean stall speed, less so in approach configuration.

Half VGs gave the best handling but a small reduction in stall

Full VGs gave good reduction of stall speed but less buffet and more wing drop than half span

The Allegro has powerful 48 degree flaps. The inboard span stalls long before the outboard and the VGs cannot help much.

Big difference in aileron response. Much nicer to fly. Although Allegro has 11m wingspan, it feels light in roll.

Surprisingly, removing the VGs close to the wing root did not bring back the buffet that was present with half span VGs

These VGs were a simple and inexpensive way to improve safety and handling

Regards

Paul