Top of Form

[image: image1.wmf]

/wEPDwUKMTc2O




[image: image3.png]7 VEYANCE




	Fuel Line/PCV/Emission Control Hose SAE 30R7

[image: image4.jpg]



Application:

A multipurpose fuel hose that can be used a fuel line, PCV connection, emission control line or fuel return line.

Specific Fuel:

Leaded and unleaded gasoline, diesel fuel, gasoline blends of ethanol (E10), E-85 and sour gas.

Construction:

Tube: Black Chemigum (NBR)
Reinforcement: 2-Spiral Synthetic
Cover: Black Hypalon* (CSM)

Temperature Range F & C: -40˚F to +257˚F (-40˚C to +125˚C)
SAE Specifications: SAE J30R7
Goodyear EP Specifications: 580019
Length: See table below. 1.6', 2' (clamshell), 25' (spool), 250' (reel)
Brand Layline: 5/16" (7.9mm) 50 psi Fuel/Emission SAE J30R7
Made in USA Goodyear
Installation: See selection of Goodyear EP clamps and connectors

WARNING: Do not use for pressure lines or fuel injected engines.

Part Number

ID(in)

OD(in)

Length (ft)

Working Pressure(PSI)

Burst Pressure(PSI)

Weight (lbs/ft)

65125

3/16

0.41

25

50

250

0.06

65122

1/4

0.50

2

50

250

0.09

65126

1/4

0.50

25

50

250

0.09

65154

1/4

0.50

250

50

250

0.09

65123

5/16

0.56

2

50

250

0.11

65127

5/16

0.56

25

50

250

0.11

65155

5/16

0.56

250

50

250

0.11

65119

11/32

0.59

1.6

50

250

0.11

65185

11/32

0.59

50

50

150

0.11

65124

3/8

0.63

2

50

250

0.12

65128

3/8

0.63

25

50

250

0.12

65156

3/8

0.63

250

50

250

0.12

65129

7/16

0.71

25

50

175

0.16

65157

7/16

0.71

250

50

175

0.16

65120

15/32

0.75

1.6

35

175

0.17

65186

15/32

0.75

50

35

175

0.17

65116

1/2

0.78

50

35

175

0.18

65121

19/32

0.91

1.6

35

175

0.24

65187

19/32

0.91

50

35

175

0.24

65117

5/8

0/94

50

35

175

0.24

65118

3/4

1.13

50

35

175

0.42

*Hypalon is registered by DuPont Dow Elastomers, LLC.

The GOODYEAR (and Winged Foot Design) trademark is used by Veyance Technologies, Inc. under license from The Goodyear Tire & Rubber Company.

Goodyear Engineered Products are manufactured and sourced exclusively by Veyance Technologies, Inc. or its affiliates.

©2008 Veyance Technologies, Inc. All Rights Reserved.


Bottom of Form

_1303542168.unknown

