Date________________

N-Number ___________

Hours _______________

KOLBRA ANNUAL INSPECTION CHECKLIST

ENGINE GROUP

______ Change engine oil (Honda motorcycle HP4 semi-synthetic w/o moly or equivalent oil).

______ Install new Rotax oil filter (3/4 turn when installing).

______ Remove spinner and check prop bolts.

______ Check prop blades for nicks, splits or other damage.

______ Check spinner for cracks.

______ Clean and re-gap or replace spark plugs.

______ Check compression if necessary.

______ Check ignition leads for security and condition.

______ Check exhaust system for cracks, leaks and security.

______ Clean or replace air cleaners and check carburetor slide valves for full throttle.

______ Check carburetors for position and security.

______ Check throttle linkage for condition and operation.

______ Check float bowls.

______ Cheek choke cables for condition and operation.

______ Check hoses for condition and security.

______ Check oil tank and hoses for condition and security .

______ Check engine shock mount assemblies and inspect rubber bushings. Replace if deteriorated.

______ Check lord engine mount weldment for cracks and distortion.

______ Check EGT and water temp and other probes and wiring for security and condition.

______ Check alternator wiring for security and condition.

______ Check all coolant lines and hoses. Replace hoses if deteriorated. Tighten all hose clamps.

______ Check coolant freeze protection and level (Evans coolant).

______ Check radiator for security and condition.

______ Check oil cooler and hoses for security and condition.

______ Check or clean/replace fuel filters.

______ Check fuel tank vents - clean.

______ Record engine hours.

FUSELAGE GROUP

______ Check rudder pedal sand brake master cylinders for security and condition.

______ Check aileron control system for condition and operation. 

______ Check flap control system for condition and operation.

______ Check elevator control system for condition and operation.

______ Lubricate all rod ends, swivel fittings, bearings and hinge bolts.

______ Check seat for fraying and secure attachment.

______ Check door hinges and latches for condition and operation.

______ Check windows and windshield for cracks and security.

______ Check fuel filler cap condition.

______ Check fuel valves for proper operation and markings.

______ Check fuel gauge on EIS for operation and proper calibration.

______ Check fuel lines for proper routing, condition and security. Replace fuel hoses at rear spar                                                                                                                                                                  if kinked or deteriorated.

______ Check instrument panel for secure mounting.

______ Check engine and flight instruments for operation and markings.

______ Check pitot system plumbing for condition and security.

______ Check instrument panel wiring for condition and security.

______ Check battery electrolyte level (if necessary).

______ Clean battery and check for secure mounting.

______ Check battery area for evidence of corrosion or acid overflow spillage.

______ Check battery wiring for condition and security.

______ Check radio installation and antennas for condition and security.

______ Check condition of fabric covering and finish.

UNDERCARRIAGE GROUP

______ Take weight off of gear legs and check for play at attach fittings.

______ Check gear legs for distortion and damage.

______ Check brake system for fluid leaks.

______ Check brake pads for wear and discs for scoring.

______ Check tires for wear and inflation (mains 14 lbs).

______ Check wheel bearings for end play and smooth rotation. Clean, inspect and repack if   wheels are removed.

______ Take weight off tail wheel and check swivel operation and grease.

______ Check tail wheel spring for distortion and damage.

______ Check spring mounting bolts for proper torque.

______ Check spring attach angles on fuselage for cracks and distortion.

______ Check tail wheel bearings for end play and smooth rotation.

______ Check steering springs and chains for condition and security.

EMPENNAGE GROUP

______ Check elevator hinge pins and bushings for excessive play.

______ Check rudder hinge pins and bushings for excessive play.

______ Check all hinge pins for condition of cotter pins.

______ Check stabilizer mounting bolts for proper torque.

______ Check stabilizer struts for distortion and damage.

______ Check strut hardware for condition and security.

______ Check fabric covering of tail surfaces.

WING GROUP

______ Remove all inspection panels and check interior of wing structure.

______ Check top surface of wing for wrinkles and irregularities that would indicate rib or                  
 spar damage.

______ Check wing tips for security and condition.

______ Check rear spar to fuselage hinge bolt and fittings for wear and security.

______ Check fronts par lock pins and fittings for wear and security.

______ Check lift strut to fuselage hinge bolts and fittings for wear and security.

______ Check lift struts for distortion and damage.

______ Check strut to spar fitting bolts.

______ Check left and right aileron hinges and condition of cotter pins 

______ Check flaperon hinge brackets for cracks and damage.

______ Check flaperon horns for condition and security.

______ Check aileron counter weight and security.

______ Check flaperon fuselage bearing.

MISCELLANEOUS

______ Check for proper display of airworthiness certificate and registration.

______ Check for proper display of “Experimental” marking and passenger warning markings.

