N1GV CONDITION INSPECTION.

Date:

Hours:

By:

Always inspect for corrosion, leaks, loose/missing fasteners, safety wire, chafing, galling and/or other significant wear.  (CAUTION: Ensure that P-lead terminals on both magnetos are grounded before working on propeller and/or engine.)  New brake pads are .190” thickness and the wear limit just prior to hitting rivets is .090”.

PROPELLER

__Remove spinner noting alignment marks

__Inspect spinner and backplate for cracks or damaged nutplates

__Check spinner mounting bolts and spacers

__Inspect propeller mounting bolts for galling, cracks and safety wire

__Check for visible grease and oil leaks at seals

__Inspect hub for cracks, corrosion and lubricate per Lubrication Chart

__Inspect propeller blades for nicks, cracks and/or surface erosion

__Check propeller track

__Reinstall spinner

POWERPLANT

[Cowling Subgroup]

__Remove cowling fasteners, hinge pins and remove upper cowling

__Remove hinge pins, fuel vent line hose and remove lower cowling

__Inspect hinges for broken eyelets, excessive hinge and/or hinge pin wear

__Clean and inspect cowling for cracks and/or distortion

__Inspect access doors, latch operation, condition and lubricate

POWERPLANT

[Lubrication Subgroup]

__Drain oil sump with engine warm and collect oil sample for analysis

__Inspect oil cooler fittings and lines for leaks and/or chafing

__Inspect oil cooler condition and door operation

__Inspect oil temperature sender for leaks and security

__Inspect oil pressure sender for leaks and security

__Inspect crankcase breather and oil separator for security and obstructions

__Drain oil separator condensate collection bowl

__Inspect propeller oil line from governor pad to front fitting for leaks

__Remove, clean and inspect oil pump suction and pressure screens

__Replace spin-on oil filter and safety wire

__Fill engine with new oil per Lubrication Chart

POWERPLANT

[Starter/Alternator Subgroup]

__Inspect Starter Pinion, Ring Gear and Grease per Lubrication Chart

__Check Balance Weight(s) Security

__Check Starter Mountings and Lead Connection

__Check Alternator Mountings and Lead Connections

__Check Alternator Condition

__Check Alternator Belt Condition and Tension

POWERPLANT

[Baffle Subgroup]

__Clean/Degrease Engine, Baffle and Airseal Gasket Surfaces

__Check Baffle Fit and Fastenings

__Inspect Baffles and Airseal Gaskets for Condition

__Seal any Gaps with RTV

POWERPLANT

[Engine/Firewall Subgroup]

__Inspect Engine Mount for Cracks

__Inspect Shock Isolators, Mounting Bolts and Washers for Condition

__Inspect Brake Fluid Reservoir Level

__Check all Firewall Penetration Seals

__Check Senders, Hoses and Wiring for Security

__Check Accessories on Rear Case for Security and Leaks

__Check Crankcase/Sump for Cracks, Leaks and/or Missing Fasteners

__Check Barrel Base Nuts for Security

__Check Rocker Box Cover Gaskets for Leaks

__Check Push Rod Covers for Straightness

__Check Push Rod Cover Seals for Leaks

__Inspect Oil Return Lines for Condition

__Inspect Exhaust System and Mounting Brackets for Cracks

__Inspect Exhaust Gaskets for Leaks and Retorque Mounting Nuts

__Check Heat Muffs, Heat Boxes and SCAT Hoses for Security

__Inspect Cylinder Head for Bent Fins and/or Burned Paint

__Pull Prop Thru and Observe Compression on each Cylinder

__Check Warm WOT Compression (PSIg) 1____ 2____ 3____ 4____

POWERPLANT

[Ignition Subgroup]

__Check Condition of Spark Plugs, Clean and Regap

__Rotate Plug Position/Polarity, Apply Anti-seize and Reinstall

__Check Ignition Harness, Springs and Insulators for Condition

__Check Magneto P-Lead Attachments

__Check Magneto Seals for Oil Leaks

__Check Breaker Felt for Proper Lubrication

__Check Magneto Points for Pitting and Correct E-Gap

__Check Magneto Distributor Block for Cracks, Burns or Tracking

__Check Impulse Coupling and Magneto Timing (BTDC) L___ R___

__Check Magneto Mounting Clamps/Nuts for Security

POWERPLANT

[Fuel System Subgroup]

__Remove and Clean Gascolator Screen and Bowl

__Clean Fuel Pump Screens

__Drain Carburetor, Clean Inlet Strainer and Retorque to 35-40 in-lb

__Inspect Condition of Fuel Lines

__Pressurize and Check Fuel System for Leaks

__Inspect Primer System for Leaks

POWERPLANT

[Intake Subgroup]

__Check Operation of Carb Heat Door, SCAT Ducting and Heat Muffs

__Check Ram Air Seal Condition

__Remove, Clean and Lubricate Air Filter

__Check Engine Controls for Proper Operation

__Check Throttle and Mixture Linkage

__Check Alternate Air, Mixture and Throttle Safety Springs

__Check Movement of all Engine Controls

__Reinstall Cowlings, Pins, Screws, Camlocs and Hinge Pin Covers

CABIN

[General Subgroup]

__Inspect Canopy Structure and Mounting Points

__Inspect Canopy Latching Mechanism and Seals

__Inspect Acrylic Window for Cracks

__Check Condition of Seats and Cushions

__Check Condition of Harnesses and Attach Points

__Check Fire Extinguisher and O2 Cylinder Condition and Currency

CABIN

[Controls Subgroup]

__Check Fuel Selector Valve Operation

__Check Cabin Fuel Lines for Leaks

__Check Elevator and Aileron Trim Servo Operation and Display

__Check Flap Operation and Display

__Check Rudder Pedal Operation and Adjustment

__Inspect Rudder Pedals, Cables and Attachments

__Check Brake Masters and Associated Tubing for Leaks

__Check Control Sticks, Linkages and Push/Pull Tubes (both ends)

__Lubricate Control Pivots and Cables

CABIN

[Instrument Subgroup]

__Check Master/Starter Contactors and Associated Wiring

__Check Master Bypass for Operation

__Check Instruments, Lines and Attachments

__Check Pitot/Static System for Leaks

__Drain Static Sump as Necessary

__Check Wing Leveler Gyro Operation

__Check Wing Leveler Servo Mounting, Linkage and Clutch

__Check Gyro Air Filter Condition and Replace as Necessary

__Clean or Replace Vacuum Regulator Garter Filter

__Check Altimeter Calibration and Operation

__Check VSI Operation

__Check Battery Box, Battery and Cables

__Check Antenna Connections and Mounting

__Check VM1000 Operation and Battery Condition

__Check ELT Operation and Battery Date (Xmitter and Remote Panel)

FUSELAGE AND EMPENNAGE

__Remove Empennage Fairings and Fuselage Inspection Plates

__Inspect Structure for Corrosion and Condition

__Inspect Engine Mount to Longeron Weldments for Cracks

__Check Control Surface Pivot Bolts for Wear

__Check Elevator Counterbalance Weights

__Check Elevator and Rudder Control Stop Condition

__Inspect Elevator Trim Tab and Actuator Rod

__Check Elevator Reversal Bellcrank, Horns and Attachments

__Check Bulkheads and Stringers for Damage

__Check Vertical Stab Spar Attach Points

__Check Rudder Horn and Attachment

__Check Horizontal Stab Spar Attach Points

__Check that Static Ports are Clear

__Check Electrical Connections

__Reinstall Inspection Plates and Fairings

__Check that all Drain Holes are Open

__Check Skin Condition and Rivets (especially the belly pan)

WING

__Remove Wing Root Fairings and Inspection Plates

__Inspect Structure for Corrosion and Condition

__Check Fuel Tanks, Lines and Drains for Leaks

__Check Fuel Vents not Blocked

__Check Fuel Cap and ‘O’ring Condition

__Inspect Wing Tip Attachment

__Inspect Wing Root Attachment

__Check Aileron System and Attach Bolts for Wear

__Check Aileron Control Stops and Counterbalance Weights

__Check Aileron Trim Tab and Actuator Rod

__Check Aileron Bellcrank Condition

__Check Flap System and Attachments for Slop

__Check Flap Torque Tube for Cracks/Distortion

__Check Flap Actuator Rod End and Jam Nut for Security

__Check Wingwalk Condition

__Check Skin Condition and Rivets

__Check Electrical Connections

__Check Pitot Tube and Plumbing

__Reinstall Inspection Plates and Fairings

LANDING GEAR

__Remove Wheel Pant Front Covers

__Check Tire Condition and Pressure

__Replace Tubes after 3 Years of Use

__Check Wheel Bearing Condition and Repack as Necessary

__Check Wheels for Cracks, Corrosion and Broken Fasteners

__Check Brake Calipers, Pads, Discs and Bushings for Wear

__Check Fluid Lines and Fittings for Security and/or Leaks

__Check Damping Strip and Gear Fairing Attachments

__Remove Nose Gear and Examine for Cracks

__Check Nose Gear Breakout Force, Stops and Grease

__Check Gear Attachment Fasteners

__Reinstall Wheel Pant Front Covers

OPERATIONAL INSPECTION

__Check Engine Driven Fuel Pump for Proper Pressure

__Check Boost Pump for Proper Pressure

__Check Fuel Selector for Engine Shutoff

__Check Oil Pressure and Temperature

__Check Alternator Output at High Draw

__Check Manifold Pressure Varies with Throttle

__Check Carb Heat Function

__Check Vacuum System

__Check Gyros for Noise and/or Roughness

__Check Altitude Reporting System and Recertify Biennially

__Check Fuel Senders/Gauges

__Check Landing, Position, Strobe, Cabin, Instrument/Warning Lights

__Check Servo Tabs and Flaps for Travel

__Check Radio Transmission, Reception and Speaker

__Check Canopy Ajar Switch

__Check Cabin Heat Operation

__Check Ignition Switch/Magneto Grounding Operation

__Check Throttle and Mixture Operation

__Check Propeller Track, Cycling and Smoothness

__Check Parking Brake Operation

__Check Brakes for Smooth Operation Both Sides

__Check VM1000 Display Elements

__Check for Proper Idle rpm and >2350 rpm Static

__Check for 20 rpm Rise upon Leaning Mixture

DOCUMENTATION

__Check Registration, Airworthiness Cert and Operating Limitations

__Check Radio Station License (Not Required for Domestic Flights)

__Check Data Plate(s)

__Review Weight and Balance Data

__Review Equipment List

__Review Airworthiness Directives for Applicability

__Review Service Bulletin/Letter Compliance

__Document Condition Inspection and Record Maintenance Performed