Assessment of G-UMPY Europa aircraft for operation under IFR or at Night against the TL2.28 Rules

Summary:

The announcement re Permit Aircraft and IFR/night was welcome and I have kept my IMC rating current in anticipation and hope to take my Europa through approval. The Europa Club have also suggested I would be well suited to be ‘first of class’.

Reviewing TL2.28 against my fit out it looks like I'd need a heated pitot and static plus– (presumably a MOD for install including on/off switch/wiring etc that will need approval, possibly using the Dynon tube in the same position).

I might fall down on instrument layout but can probably find an acceptable arrangement (a Sky Demon map being central to the panel rather than a six pack arrangement).

However, regarding an Attitude Indicator (AI) it is disappointing that the Trutrak Gemini Auto Pilot with integral Primary Flight Display that I have installed is uniquely singled out as being, unsuitable. Quote from TL2.27 (ref 1)

Instruments which are NOT acceptable to the LAA to provide required instruments for IMC flight: Manufacturer Instrument types …

TruTrak ADI (inc ADI pilot) Gemini EFIS These instruments provide misleading indications in pitch and roll, particularly at low speed, and are not to be fitted to aircraft approved for operation in IMC, under IFR or at night.

Possibly the rationale for this is that discussed in the trutrak website (Ref 2 relevant text included below), although perhaps it has come from adverse experience during the trials on the first 4 IFR LAA aircraft.

Having converted from a vacuum AI to the Gemini PDF a season ago, and flown IMC previously for many years. I have found it as good or better and (to my surprise actually) exceedingly usable and well behaved as also is the autopilot it drives (the instrument needs correct, robust information for controlling the pitch and roll servos). So am puzzled by this and will discuss it with Trutrak inc.

I was expecting to require a second AI (possibly the return of the vac AI) and I would like to know before progressing further if installing a vac AI to centre view and relegating the Gemini off centre with a label (something like VSI and BANK angle shown) then I might have a way forward. Otherwise I can’t see a way to progress it without losing a significant investment in the AP.

# Preliminary Assessment

There follows a preliminary assessment to see where the issues might be with tendering an application. Table 1: Assessment of Europa G-UMPY against the rule set is provided below as an initial view.

| Rule | Theme | Statement | Notes/evidence |
| --- | --- | --- | --- |
| 1.1.1 | Flying Hours | Compliant | 250hrs |
| 1.1.2 | Previously certified | Compliant | N/A |
| 1.2.1 | Engine Type | Compliant | Rotax 912ULS |
| 1.2.2 | Previous PTC | Compliant | N/A |
| 1.2.3 | Carb heating system | Compliant | None mandated by manufacturer |
| 1.2.4 | Engine modifications | Compliant | Non fitted |
| 1.2.5 | Ignition system | Compliant | Standard Rotax 912ULS |
| 1.2.6 | Propeller edge | Compliant | Woodcomp 3000/3 with plastic protected leading edge section |
| 1.3.1 | Unapproved instruments | Compliant | All instruments designed for purpose |
| 1.3.2 | VFR flight Instruments ASI, ALT and Compass | Possibly Compliant | Note ALT 10,000 ft single pointer |
| 1.3.3 | Power plant instruments | Compliant | See table 1 |
| 1.3.4 | IMC Instruments | Compliant | See table 1, layout may need revision, Vacuum AI Turn coordinator provided by PFD DI provided by PFD, GPS, Skydemon VSI provided by AI, PFD, Skydemon, GPS AI and PDF different types and one unpowered |
| 1.3.5 | Instrument layout | Compliant | AI central, ASI to left, ALT to below left, PDF (providing VSI, to top right), DI provided by Skydemon below |
| 1.3.6 | Navigation systems | Compliant | ETSO approved radio ATR833 and transponder TRT800H (both funkwerk) Garmin 500 providing positional information to transponder |
| 1.3.7 | Display of radio nav data | Compliant | Radio and transponder have their own displays. General navigation data available from Aera500 and skydemon on nexus android tablet both independently self powered (4hrs and 11hrs both charged from main bus) |
| 1.3.8 | Control of comm radios | Compliant | Radio has its own OLED display and is not controlled via a complex instrument |
| 1.3.9 | Outside air temp | Compliant | Dedicated gauge measuring air in plenum chamber |
| 1.3.10 | Heated pitot tube | Non-compliant | Dynon heated type including heated static port to be fitted to replace existing as part of a MOD application |
| 1.3.11 | Static source | Non-compliant | To be included in the above |
| 1.3.12 | Check of installed systems | Test to be performed | Not required for the initial application |
| 1.3.13 | In flight systems check | Test to be performed | Needs to be done prior to application (with pitot mod applied presumably) |
| 1.3.14 | Autopilots | Compliant | Trutrak Gemini AP installed controlling pitch and roll, route provided by Garmin Aera500 GPS. Independent of vac AI |
| 1.3.15 | Warnings | Compliant | Warnings on Skydemon and Aera500 all cancellable and configurable |
| 2.3 | Battery Capacity | Compliant | See table 2 for load budget. Odyssey PC680 battery fitted 18AH capacity. Load with autopilot disengaged 4.46A; expiry 4.5hrs. Alternator failure indicated with LED plus negative charge current on V/I guage |
| 2.4 | Electrical dependence | Compliant | LED driven when charging fails. Main AI driven from vacuum, engine does not require battery power |
| 2.5 | Radio power supply | Compliant | Only single radio installed (listen on two channels). Handheld available as backup |
| 2.6 | Electric trim system | Compliant | Gemini AP controls trim when servos are active, disengaging AP causes trim control to be disengaged from AP via a multipole relay in the trim module and reconnects to switches. |
| 2.7 | Electric flap | Compliant | Flaps are manually operated |
| 2.8 | Electronic control systems for main bus/power | Compliant | No electronic control |

Table 1: Assessment of Europa G-UMPY against the rule set

# Information required/available

The following information is required to accompany the TL2.28 application

|  | Artifact | Status |
| --- | --- | --- |
| 1 | Instrument layout photograph | Can be provided: AI needs adding back in Gemini PFD moving left Area500 moving to old iphone map position (left and down) |
| 2 | ASI in flight calibration chart | Available from initial flight test, can be performed, requires a flight. New pitot may affect which is dependent on MOD definition/application/approval |
| 3 | Electrical load analysis report | Available: Table 4 Power Budget |
| 4 | Weight and Balance report | Available |
| 6 | Tailored Maintenance Schedule | Available – will need updating |
| 7 | Pilot Operating Handbook | Available for standard mono – will need work to update, use the POH lite format |
| 8 | Modification Summary | Available in back of airframe log book |

Table 2: Information required with the application

# Review of Information, Instrumentation, duplication and Backup

Table 3 shows the information down the left hand side required to fly safely or available. Across the top are listed the instruments that are installed. Those listed in black are those that do not require main bus power, those in green have internal batteries and function without main bus power for a considerable period of time. Those listed in red require main bus power to function.

The matrix shows the duplicated sources of information.  Those instruments marked ‘$’ are stand alone gauges such as oil temperature or fuel quantity. There follows a failure analysis to check that safe flight can be conducted in the event of loss of an instrument or feed, for example pitot pressure.

Key to Table 3

Powered

Independently Powered

Unpowered

ASI - Witam air speed indicator

ALT - Falcon altimeter single indicator (10,000 ft mb)

RPM - Rotax type

PDF Trutrak Gemini combined PFD and autopilot

AI artificial Horizon vac driven

GPS Garmin Aera500

SkyD Sky Demon running on a 7"nexus android tablet

SA-3 smart Ass audio low airspeed/wing loading warning

Stall warner - europa standard

FL-5 IE fuel flow/totaliser

TRT TRT800 transponder OLED Funkwerk with ADS-B in

ATC ATR833 transceiver OLED Funkwerk

PFLARM - traffic avoidance inc mode S/C ADS-B

$ dedicated instrument/gauge

| Information | ASI | ALT | Compass | RPM | PFD | AI | Slip | GPS | SkyD | Smart Ass3 | Stall | CS-1 | FL-5 | TRT | ATC | PFLARM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airspeed | x |  |  |  | x |  |  | x | x | x |  |  |  |  |  |  |
| Ground Speed |  |  |  |  | x |  |  | x | x |  |  |  |  |  |  |  |
| Height |  | x |  |  | x |  |  | x | x |  |  |  |  |  |  |  |
| Attitude |  |  |  |  | x | x |  | x |  |  |  |  |  |  |  |  |
| Slip $ |  |  |  |  | x |  | x |  |  |  |  |  |  |  |  |  |
| Veritical Speed |  |  |  |  | x | x |  |  | x |  |  |  |  |  |  |  |
| Heading |  |  |  |  | x |  |  | x | x |  |  |  |  |  |  |  |
| Track |  |  | x |  |  |  |  | x | x |  |  |  |  |  |  |  |
| Bank angle |  |  |  |  | x | x |  | x |  |  |  |  |  |  |  |  |
| Outside Air Temperature $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |
| Wing loading |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |
| Stall Warning $ |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |
| RPM |  |  |  | x |  |  |  |  |  |  |  | x |  |  |  |  |
| Manifold Pressure |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
| Oil Pressure $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oil Temp $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Head Temp $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Remaining $ $ |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
| Fuel Flow |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |
| Fuel Pressure $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternator Health $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge Current $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Battery Voltage $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Trim motor active $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Trim state $ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Propeller Pitch |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |
| Map/position $ |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |
| Frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |
| Sqwark |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |
| Traffic Position |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |

Table 3: Information against powered and unpowered sources

# Failure Analysis

The following list considers the effect of various faults on safe flight and shows that there is no single fault that could occur and make critical information available. Attitude, Speed, height could be considered to be the most critical. Fuel state is available from a level gauge, in cockpit sight tube and totaliser/flow instrument.

- Loss of ASI instrument: Air speed from PFD, spoken speed on approach from Smart Ass-3 instrument.

- Loss of ASI pressure; Ground speed from PDF, GPS, Skydemon, Air speed estimated by factoring wind speed/angle

- Loss of static pressure or ALT instrument: Altitude available from PFD and GPS

- Loss of Slip ball; yaw available from PFD

- Loss of compass; heading and track available from PFD, GPS and SkyD

- Loss of AI: attitude available from PFD, turn from slip ball, GPS, VSI from PFD, GPS and SkyD

- Loss of CS-1 propeller controller;  pitch remains as last set, manifold pressure relative to throttle position, RPM from dedicated Rotax gauge

- Loss of FL-5 fuel flow instrument; fuel level available from sight tube between seats and dedicated fuel level gauge

- Loss of power to PFD: airspeed, attitude, height, slip, heading  available from dedicated instruments and self powered  GPS and SkyD (on android tablet)

- Loss of GPS (feeds autopilot route tracking): position/mapping available from independently powered SkyD. Autopilot works on independent gyros to lock track and height.

- Loss of transponder: not safety critical

- Loss of two channel ATR833 Funkwerk radio: hand held transceiver backup.

- Loss of charging (indicated by charging light off, negative current draw on V/I gauge:  Autopilot control off to minimise and current possibly transponder,

Power Budget

Table 4 shows the equipment that is powered from the main 12V bus and from the Rotax alternator which is capable of providing a maximum of 18 amps when the engine is running at 5000 rpm. It shows that even with the auto pilot servos operating (stepper motors) there is adequate margin. The battery capacity is 18 AHR. If charging was lost in IMC when the autopilot was on then it could be expected that the battery would hold out for more than an hour. If the AP was disengaged than the battery should hold up for several hours and in any case it is not critical for safe flight.

| Source/drain | Amps | Comment |
| --- | --- | --- |
| Rotax 912ULS current supply max | 18 |  |
| GPS Charging | 0.5 | Estimate self powered |
| NEXUS charging | 0.5 | Estimate self powered |
| Fuel Flow FP-5L EI | 0.1 |  |
| Trutrak Gemini | 0.5 |  |
| Auto Pilot servos | 4 |  |
| ATR883 | 0.15 | 1.8 TX |
| TRT800H | 0.3 |  |
| PFLARM | 0.165 |  |
| Strobe | 1 | estimate |
| Fuel pump | 0 | estimate 1.4 when on |
| Fuel meter | 0.05 |  |
| Instruments | 0.2 |  |
| Battery charging | 1 |  |
| Static port heater? | 1 | 12Watts |
| total | 9.465 |  |

Table 4 Power Budget

# References/Notes

Ref 1. Page 38, http://www.lightaircraftassociation.co.uk/engineering/TechnicalLeaflets/Operating%20An%20Aircraft/TL%202.28%20Night%20IFR%20assessment.pdf

Ref.2 http://www.trutrakap.com/faq/ relevant text included below.

Q: Difference between the ADI turn information and a conventional AI

The bank angle display is driven in a manner similar to that used in the pictorial turn and bank. The only difference is that in this instrument the display shows bank angle instead of turn rate. The advantage of this is that the presentation is instantaneous data only. It cannot drift to accumulate error as is possible with the more complex AHRS driven systems. It shows bank angles of up to +/- 45° while the requirement on conventional gyro horizons is +/- 100°. To compensate for this, flashing red arrows point in the direction the stick is to be moved to level the aircraft when the bank exceeds +/- 30°. Note that when flying on instruments the object is to stay level or at least in shallow banks. This instrument is therefore designed to keep the pilot out of trouble.

Q: Difference between ADI Pitch Display and a conventional AI

The pitch display is perhaps the most beneficial feature of the ADI when compared with conventional horizon indicators. For short term or immediate movement of the nose up or down, this instrument responds and is flown like any other gyroscopic pitch indicator. The difference is that after the display has moved in response to an attitude change the deflection is sustained by the resulting vertical speed. As compared to a pitch attitude display, the vertical speed display makes it easier to hold altitude and to maintain stable climbs and descents. Also, the movable airplane symbol does not have to be adjusted to compensate for the angle the fuselage flies when vertical speed is zero. It is recognized that an aircraft when at the limit of its altitude capability can fly at an ever increasing angle of attack when vertical speed indicates zero. For this reason an airspeed warning in the form of the letters “A-S” flashing in the DG display is provided. The answer to this situation is then to observe the airspeed indicator. For stall prevention airspeed is the best information short of an angle of attack indicator. (Better than attitude)