Garmin Initial Autopilot Settings

- RV-10

  - Roll Servo

    - Max Torque = 100%

    - Servo Gain = 0.80

    - Fine Adjust Time = 0.20

    - Fine Adjust Amount = 0

  - Pitch Servo

    - Max Torque = 100%

    - Servo Gain = 2.50

    - Fine Adjust Time = 0.20

    - Fine Adjust Amount = 1

  - Pitch Gain

    - Min Airspeed Limit = 70 kt

    - Max Airspeed Limit = 180 kt

    - Vertical Speed Gain = 1.40

    - Vertical Accel Gain = 1.00

    - Airspeed Gain = 1.35

    - Airspeed Accel Gain = 1.00

  - Manual Electric Trim

    - Airspeed Threshold = 80 kt / 150 kt

    - Roll Trim Motor Speed = 100% / 50%

    - Pitch Trim Motor Speed = 100% / 50%