# Wiring of the S704 in Z101

The Z101 diagram has the following detail in the upper left – I have added letters A-E to name the fast-on terminals.

B&C publishes directions for installing the S704-1, as follows:

Here is how I read the cross-mapping:

A = Com in this case a feed over to the engine bus

B = the coil negative

C = the coil positive

D = NO, the normally open terminal, in this case connected from the fat wire tie point

E = NC, the normally closed terminal.  Here, unused, as there is no flow of current until the coil fires and we get the alternate power feed through D to A and into the engine bus.

B&C are connected by a diode and C&D are jumpered